#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#define TRIG_PIN 9

#define ECHO_PIN 10

#define BUZZER_PIN 8

#define GAS_PIN A0

LiquidCrystal_I2C lcd(0x27, 16, 2);

unsigned long previousBuzzerMillis = 0;

unsigned long previousLCDMillis = 0;

bool buzzerState = false;

// ── Thresholds ────────────────────────────────

#define GAS_DANGER 25

#define GAS_WARNING 18

#define DIST_TOCLOSE 10

#define DIST_NEAR 30

#define DIST_APPROACH 80

void setup() {

Serial.begin(9600);

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

pinMode(BUZZER_PIN, OUTPUT);

digitalWrite(BUZZER_PIN, LOW); // Force OFF at start

lcd.init();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print(" Smart Detector ");

lcd.setCursor(0, 1);

lcd.print(" Initializing..");

delay(3000);

lcd.clear();

}

long getDistance() {

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

long duration = pulseIn(ECHO_PIN, HIGH, 30000);

if (duration == 0) return -1;

return duration * 0.034 / 2;

}

// ── Buzzer Handler (Active Buzzer) ────────────

void handleBuzzer(int gasValue, unsigned long currentMillis) {

int buzzerInterval = 0;

if (gasValue > GAS_DANGER) {

buzzerInterval = 100; // Fast beep

} else if (gasValue > GAS_WARNING) {

buzzerInterval = 350; // Medium beep

}

if (buzzerInterval > 0) {

if (currentMillis - previousBuzzerMillis >= (unsigned long)buzzerInterval) {

previousBuzzerMillis = currentMillis;

buzzerState = !buzzerState;

// Active buzzer uses digitalWrite not tone()

digitalWrite(BUZZER_PIN, buzzerState ? HIGH : LOW);

}

} else {

// Safe — buzzer fully OFF

digitalWrite(BUZZER_PIN, LOW);

buzzerState = false;

previousBuzzerMillis = currentMillis;

}

}

void updateLCD(int gasValue, long distance) {

// ── Row 0: Distance ───────────────────────

char row0[17];

if (distance == -1) {

sprintf(row0, "D:-- cm OUT RNG");

} else if (distance < DIST_TOCLOSE) {

sprintf(row0, "D:%-4ldcm TOCLOSE ", distance);

} else if (distance < DIST_NEAR) {

sprintf(row0, "D:%-4ldcm NEAR ", distance);

} else if (distance < DIST_APPROACH) {

sprintf(row0, "D:%-4ldcm APPROACH", distance);

} else {

sprintf(row0, "D:%-4ldcm CLEAR ", distance);

}

lcd.setCursor(0, 0);

lcd.print(row0);

// ── Row 1: Gas ────────────────────────────

char row1[17];

if (gasValue > GAS_DANGER) sprintf(row1, "G:%-4d !!DANGER!!", gasValue);

else if (gasValue > GAS_WARNING) sprintf(row1, "G:%-4d WARNING ", gasValue);

else sprintf(row1, "G:%-4d SAFE ", gasValue);

lcd.setCursor(0, 1);

lcd.print(row1);

}

void loop() {

unsigned long currentMillis = millis();

long distance = getDistance();

int gasValue = analogRead(GAS_PIN);

// ── Buzzer ────────────────────────────────

handleBuzzer(gasValue, currentMillis);

// ── LCD + Serial every 300ms ──────────────

if (currentMillis - previousLCDMillis >= 300) {

previousLCDMillis = currentMillis;

Serial.print("Gas: ");

Serial.print(gasValue);

Serial.print(" | Dist: ");

if (distance == -1) Serial.println("Out of range");

else { Serial.print(distance); Serial.println(" cm"); }

updateLCD(gasValue, distance);

}

}